Novus Translation Document

Match bold with bold

|  | English | Your Language |
| --- | --- | --- |
|  | This video is not intended to replace the Instructions for Use that are supplied with each Novus™ mask, always refer to the instructions provided with each mask before use. |  |
| 01 | novus™ nasal CPAP mask |  |
| 02 | How to fit |  |
| 03 | Intersurgical. Complete respiratory systems |  |
| 05 | Remove the swivel connector from the mask by depressing the quick release tabs. |  |
| 06 | Attach the connector to your device tubing. |  |
| 07 | Place the mask seal over the nose and holding in position, gently pull the headgear over your head. |  |
| 08 | Ensure that the lower straps are below the ears. |  |
| 09 | Check that the straps are not twisted and engage the remaining lower strap onto the mask hook. |  |
| 10 | Adjust the mask fitting. The fit should be secure but not over tight. |  |
| 11 | Ensure that the CPAP / bilevel device is operating, air should be flowing from the device tubing. |  |
| 12 | Attach the assembled swivel connector and device tubing to the mask elbow. |  |
| 13 | Removal |  |
| 14 | Disengage one of the lower straps and pull the headgear over your head. |  |
| 15 | Disassembly |  |
| 16 | Release the upper and lower headgear straps. |  |
| 17 | If fitted, the quick release headgear clip can remain attached to the headgear. |  |
| 18 | Disconnect the assembled device tubing and swivel connector by depressing the quick release tabs. |  |
| 19 | Gently pull and twist the swivel connector to remove it from the device tubing. |  |
| 20 | Carefully remove the seal from the mask shell. |  |
| 21 | If fitted remove the forehead spacer by depressing the two side clips. |  |
| 22 | Assembly |  |
| 23 | Align the seal interface channel to the rear edge of the mask shell. |  |
| 24 | Gently press each section of the seal interface onto the mask seal. |  |
| 25 | If required, fit the forehead spacer. |  |
| 26 | Re-attach the headgear. |  |
| 27 | Assemble the swivel connector to the device tubing and then attach the swivel connector to the mask elbow. |  |
| 28 | Check to ensure that all component parts are undamaged and functional prior to use. |  |