Nebuliser video

Please put in bold where stated

|  | English | Your Language |  |
| --- | --- | --- | --- |
| 1 | Intersurgical Nebulisers |  |  |
| 2 | Airway Management Critical Care Anaesthesia Oxygen and Aerosol Therapy |  |  |
| 3 | Intersurgical is Europe’s leading designer, manufacturer and supplier of a wide range of medical devices for respiratory support. |  |  |
| 4 | We have extensive experience designing, manufacturing and supplying a wide range of nebuliser products, which generate a clinically effective drug aerosol. |  |  |
| 5 | Quality is designed into all aspects of our products, from concept through to final manufacture, to ensure product integrity. |  |  |
| 6 | The Intersurgical in-house laboratory facility which is used for nebuliser product development and quality control, measures nebuliser performance charatersistics in line with ISO standard 13544-1. |  |  |
| 7 | The Malvern spraytec machine Measures the characteristics of the aerosol generated, such as the mass median diameter and the nebuliser efficiency, to ensure our nebulisers have a consistent performance and deliver a clinically effective aerosol. |  |  |
| 8 | Definitions Mass Median Diameter (MDD) Half of the mass of the nebuliser output is contained in droplets which are larger than the MMD and the other half of the massis contained in droplets smaller than the MMD. Nebuliser efficiency # Is the percentage of particles of the optimum size (<5 microns). |  |  |
| ## 9 | ## Further performance characteristics are determined, including the nebuliser ## output and back pressure. Definition Nebuliser aerosol output Mass output per minute of drug particles in an aerosol. |  |  |
| ## 10 | ## The Cirrus™2 and HOT Top™2 nebulisers are designed to generate an aerosol of 2-5 microns for tracheobronchial deposition. |  |  |
| 11 | Particles smaller than 0.5 microns expired back to atmosphere |  |  |
| 12 | Particles above 5-10 microns impact on the upper respiratory tract |  |  |
| ## 13 | Cirrus™2 nebuliser is a closed jet nebuliser designed to generate an aerosol for tracheobronchial deposition. |  |  |
| ##### 151 | ## HOT Top™2 nebuliser ## generates an aerosol for tracheobronchial deposition, with an open vent design which increases output flows, reducing treatment times and improved patient compliance. |  |  |
| ## 16 | ## Interfaces ## A range of nebulisers are available with a range of patient interfaces ## including the Intersurgical EcoLite™ aerosol masks. |  |  |
| ## 17 | ## A choice of "under chin or "on chin" positions ## Intersurgical EcoLite™ mask provides a better fit on a wider range of patient face shapes. |  |  |
| ## 18 | ## Incurved nose ## Conforms to different nose shapes designed to prevent aerosolised drug entering patient’s eyes. |  |  |
| 19 | Performance The improved mask design prevents ## drug wastage. |  |  |
| 20 | Interfaces Available with Intersurgical Mouthpieces. |  |  |
| ## 21 | ## Interfaces ## Nebuliser T-piece kits are available ## for use in breathing systems, including the self sealing T-piece. |  |  |
| 22 | The Intersurgical nebuliser testing facility is used for product development and quality control of Intersurgical nebulisers. |  |  |
| 23 | Product design Testing capability Manufacturing facilities Quality control |  |  |
| 24 | Intersurgical Nebulisers Intersugical's in-house experience and specialised facilities all contribute to producing a consistent, high performance range of nebulisers, optimised for tracheobronchial deposition to provide a clinically effective treatment every time. |  |  |
| 25 | Intersurgical Quality, innovation and choice Interact with us |  |  |