|  | # English | # (your language) |
| --- | --- | --- |
| 01 | The AbCanTM Disposable carbon dioxide absorber |  |
| 02 | The AbCan Intersurgical’s easy to use disposable absorber that has been developed specifically for clinical use during anaesthesia to absorb carbon dioxide within an anaesthetic breathing system |  |
| 03 | AbCan Spherasorb, white to violet AbCan Spherasorb, pink to white AbCan LoFloSorb, green to violet |  |
| 04 | Fast and easy exchange The AbCan enables fast exchange of absorbent without the need for time consuming pouring of loose granules, or the risk of contamination |  |
| 05 | Medical grade absorbents for extra safety with volatile anaesthetics |  |
| 06 | Air-tight seal for storage |  |
| 07 | No dust emission Low resistance scrim membranes ensure that any fine particles or dust are retained within the AbCan and not allowed to escape into the breathing system or into the atmosphere during handling |  |
| 08 | Efficient CO2 absorption A gas dispersion chamber at the base of the AbCan allows for optimum flow up through the absorbent |  |
| 09 | Fitting the AbCanTM |  |
| 10 | Remove twin seal cap |  |
| 11 | Before fitting ensure that the granules inside the AbCan are settled and level. This is to provide optimum performance. This can b achieved by a few gentle taps. |  |
|  | Insert the AbCan absorber ports up into the corresponding machine ports so that the AbCan sits vertically. |  |
| Tags |  |  |
|  | Insert the AbCan absorber ports up into the corresponding machine ports so that the AbCan sits vertically. |  |
| 22mm self sealing T-piece |  |  |
|  | English | (your language) |
| 01 | The AbCanTM Disposable carbon dioxide absorber |  |
| 02 | The AbCan Intersurgical’s easy to use disposable absorber that has been developed specifically for clinical use during anaesthesia to absorb carbon dioxide within an anaesthetic breathing system |  |
| 03 | Green to Violet White to violet Pink to white |  |
| 04 | Fast and easy exchange The AbCan enables fast exchange of absorbent without the need for time consuming pouring of loose granules, or the risk of contamination |  |
| 05 | Medical grade absorbents for extra safety with volatile anaesthetics |  |
| 06 | Air-tight seal for storage |  |
| 07 | No dust emission Low resistance scrim membranes ensure that any fine particles or dust are retained within the AbCan and not allowed to escape into the breathing system or into the atmosphere during handling |  |
| 08 | Efficient CO2 absorption A gas dispersion chamber at the base of the AbCan allows for optimum flow up through the absorbent |  |
| 09 | Fitting the AbCanTM |  |
| Title | Remove twin seal cap |  |
| Description | Before fitting ensure that the granules inside the AbCan are settled and level. This is to provide optimum performance. This can be achieved by a few gentle taps. |  |
|  | Insert the AbCan absorber ports up into the corresponding machine ports so that the AbCan sits vertically. |  |
| Tags |  |  |
|  | Insert the AbCan absorber ports up into the corresponding machine ports so that the AbCan sits vertically. |  |

| 12 | Insert the AbCan absorber ports up into the corresponding machine ports so that the AbCan sits vertically. |  |
| --- | --- | --- |
| 13 | Now turn the locking switch so it is pointing in the upward locked position. |  |
| 14 | The AbCan is now raised into the locked position and is ready for the pre-use leakage and compliance test. |  |
| 15 | If using a filter on the inspiratory port of the machine, to avoid increased resistance to flow, place an additional water trap (1915000) before the filter. 1915000, Water trap, self-sealing, right-angled, high volume |  |
| 16 | In addition, to allow easier positioning of the AbCan canister it is recommended to use a spacer, such as 1961000 straight 22M-22F between the inspiratory port and the water trap. 1961000, Straight connector 22M-22F |  |
| 17 | Use the filter according to its intended use, and replace as indicated in the filter IFU. |  |
| 18 | Use push and twist action during connecting and disconnecting system components for safe and secure connections. |  |
| 19 | Turn the locking switch so it is pointing in the downward unlocked position and remove the AbCan. |  |
| 20 | WARNING: If the FLOW-i machine has a Bypass position on the locking switch, the AbCan will not absorb patient CO2 with the locking switch in this position. |  |
| 21 | Getinge is a registered trademark of Getinge AB. Maquet is a registered trademark of Maquet GmbH. Flow-i, Flow-c and Flow-e are trademarks for Maquet Critical Care AB |  |
| 22 | Always refer to the instructions for use provided in each box before using the AbCanTM |  |
| 23 | # Quality, Innovation, and Choice Interact with us |  |
| Youtube Title | # The AbCan™ The AbCan™1.1L pre-filled disposable carbon dioxide absorber |  |
| Description | # The AbCan™ offers ease of exchange with minimal handling and provides a further option for customers who have made the choice to use pre-fills. The AbCan™ is provided with a choice of, Spherasorb™ medical grade soda lime, or LoFloSorb™, medical grade alkali-hydroxide-free absorbent. This provides the user with a choice of absorbent according to their requirements and preference. The AbCan™ is compatible with the Getinge® (formerly Maquet®) Anesthesia Delivery Systems, Flow-I™ C20, C30, C40, Flow-C™ and Flow-E™. # Getinge is a registered trademark of Getinge AB. Maquet is a registered trademark of Maquet GmbH. FLOW-I, Flow-C and Flow-E are trademarks for Maquet Critical Care AB. |  |