| 01 | BiTrac®-Select-NIV-Full-Face-MaskFitting-Guide |  |
| --- | --- | --- |
| 02 | This video is a brief introduction to the BiTrac® Select NIV Full Face Mask. BiTrac® Select NIV Full Face Mask must always be used in accordance with the Instructions for Use supplied with the device. This video is not a replacement for the Instructions for Use. Intersurgical accepts no responsibility for the medical practices of those who directly or indirectly reference this video. |  |
| 03 | BiTrac® Select NIV Full Face Mask |  |
| 04 | Fitting Guide |  |
| 05 | The BiTrac range of NIV masks includes total face, full face and nasal mask options. An interface rotation strategy may help to reduce or prevent patient discomfort associated with prolonged NIV application. |  |
| 06 | The BiTrac® Select interchangeable elbow range can greatly reduce waste. You can easily remove and replace the mask’s supplied elbow with separately available elbows designed for transitioning patients between treatments. |  |
| 07 | Three BiTrac® Select Standard Elbows |  |
| 08 | Non-vented for dual limb breathing systems |  |
| 09 | Vented with anti-asphyxiation valve for single limb breathing systems that have no CO₂ leak port |  |
| 10 | Non-vented with anti-asphyxiation valve with anti-asphyxiation valve for single limb breathing systems that have a CO₂ leak port |  |
| 11 | Two BiTrac® Select Bronchoscopy Elbows |  |
| 12 | Non-vented Bronchoscopy for dual limb breathing systems |  |
| 13 | Non-vented Bronchoscopy with anti-asphyxiation valve for single limb breathing systems that have a CO₂ leak port |  |
| 14 | Two BiTrac® Select Nebulisation Elbows for use with full face mask only |  |
| 15 | Non-vented nebulisation for dual limb breathing systems |  |
| 16 | Non-vented nebulisation with anti-asphyxiation valve for single limb breathing systems that have a CO₂ leak port |  |
| 17 | The BiTrac® NIV Full Face Mask is highly adjustable, and the design minimises the pressure exerted on the bridge of the nose. |  |
| 18 | BiTrac® Select NIV full face masks with interchangeable elbows |  |
| 19 | 4 sizes available from extra large adult to small adult |  |
| 20 | Dual lip silicone seal for patient comfort and airtight fit |  |
| 21 | Bayonet style lock for secure elbow connection |  |
| 22 | Secure clips |  |
| 23 | Additional Velcro tabs Each strap includes two Velcro tabs for optimal patient fit |  |
| 24 | Interchangeable elbows available to fit your patient’s needs |  |
| 25 | Breathable headgear |  |
| 26 | Adjustable Omni-Clip™ for optimal forehead pad adjustment |  |
| 27 | Fitting |  |
| 28 | Use the size guide provided to measure the patients face. It should fit on the bridge of the nose, and below the lower lip. |  |
| 29 | If the size guide provided is a sizing card. The measurement taken is the same. On the bridge of the nose and below the lower lip. |  |
| 30 | Take the correct sized mask, grip the spindle and attach the desired bayonet elbow. |  |
| 31 | Check the smooth operation of the spindle. |  |
| 32 | Changing the elbow for each desired treatment can be done simply at any time, even on the patient. |  |
| 33 | Place the mask on the patients face. Then slip the headgear over the back of the patient’s head. |  |
| 34 | Adjust the top strap. |  |
| 35 | Attach the bottom clips of the headgear to the mask. There should be an audible click. |  |
| 36 | Adjust the top Velcro straps accordingly. Do not over tighten the mask. |  |
| 37 | Then adjust the bottom Velcro straps. |  |
|  | Ensure the cushion is still above the eyebrow and below the lower lip. |  |
|  | Start the gas flow and attach the breathing system. |  |
|  | Mask position and headgear can be adjusted at any time. |  |
| 39 | Removal |  |
| 40 | Detach the clips and lift away. |  |
| 41 | Intersurgical |  |
| 42 | Quality, innovation and choice |  |
| 43 | Interact with us |  |
| 44 | www.intersurgical.com |  |