| 01 | Dri-ThermTM dual heated wire breathing system Reduced condensate technology |  |
| --- | --- | --- |
| 02 | The Dri-Therm™ heated wire breathing system is designed to reduce the humidity of gas in the expiratory limb, keeping tubing clear of fluid, helping maintain a closed system and protecting the ventilator from high levels of humidity. |  |
| 03 | Condensation is a common issue when humidifying mechanically ventilated patients in the ICU. |  |
| 04 | Condensation forms when the warm, humid gas inside the breathing system cools and the humidity held within the gas condenses on to the internal surfaces as water. |  |
| 05 | Cooler air outside tube |  |
| 06 | Condensation can occur in most areas of an active humidification breathing system; however, the accumulation of water in the expiratory gas pathway can be particularly problematic. |  |
| 07 | Expiratory limb |  |
| 08 | Ventilator |  |
| 09 | Expiratory filter |  |
| 10 | If allowed to accumulate, condensate can interfere with the delivered breath and pose a risk to patient safety. Intervention is required to break the circuit and remove the excess water. |  |
| 11 | Just a small amount of condensation can cause a disruption to the function of sensors and valves. |  |
| 12 | Condensate inside the filter can lead to an increase in resistance and an increase in patient Work of Breathing. |  |
| 13 | The Dri-Therm™ heated wire breathing system is designed to reduce the humidity of gas in the expiratory limb, keeping tubing clear of fluid, helping maintain a closed system and protecting the ventilator from high levels of humidity. |  |
| 14 | Dri-Therm expiratory tubing is made of material that allows small water molecules to pass through the limb wall. The permeability of the tube removes excess condensation. |  |
|  | Larger bacteria and viruses cannot pass through the tubing and are contained within the breathing system so that they can be safely filtered. |  |
| 15 | Water vapour |  |
| 16 | Condensate |  |
| 17 | Bateria |  |
| 18 | Virus |  |
| 19 | The Dri-Therm system combines permeable tube technology with Intersurgical’s twin coil wire and high quality product design. |  |
| 20 | The temperature and humidity of the gas is conditioned to deliver the optimal level of humidification to the patient whilst minimising excess condensate in the gas pathway. |  |
| 21 | Intersurgical’s transparent, lightweight tubing and connectors ensure complete visibility of the breathing system patency and performance throughout patient treatment. |  |
| 22 | Benefits for the patient |  |
| 23 | Benefits for the clinician |  |
| 24 | Improved ventilator performance |  |