Intersurgical i-flow interFlow™

|  | English |  |
| --- | --- | --- |
| 1 | WARNING: This set-up video is not intended to replace the device Instructions For Use (IFU). Read and familiarise yourself with the device, and all associated device IFUs before first use. |  |
| 2 | Quick set up guide |  |
| 3 | Display Power button Proximal pressure port O2 supply connection Respiratory gas outlet Air supply connection |  |
| 4 | You will need the following equipment InterFlow Humidifier Air and oxygen hoses Equipment stand |  |
| 5 | Connect the air and oxygen hoses to InterFlow’s gas inlets. Connect the hoses to the air and oxygen wall outlets. Connect to the mains power supply. |  |
| 6 | Device Start-up To switch on the device, press and hold the power on/off button. |  |
| 7 | Therapy Screen Press the mode symbol on the therapy display screen to change the selected therapy mode. Use the navigation arrows to cycle through the therapy mode options. |  |
| 8 | Press the tick symbol then conform the selection. |  |
| 9 | Treatment To start treatment, press the start treatment symbol on the therapy screen. To change the therapy parameter setting, press the parameter value on the therapy screen. |  |
| 10 | Therapy Parameter Adjustment Press - or + to adjust the value. Press the tick symbol to confirm selection and return to the therapy screen. |  |
| 11 | Repeat the same process to adjust the O2 parameters To start treatment, press the start treatment symbol on the therapy screen and again to confirm. |  |
| 12 | Therapy Graph Display To change the graph information displayed, press the graph on the therapy screen to cycle. |  |
| 13 | Switching Off To switch off the device, press the power on/off button. To confirm selection, press the power on/off symbol on the screen. |  |
| 14 | Intersurgical Quality, innovation and Choice Interact with us www.intersurgical.com |  |