VIDEO  CP1170   (kit including the NIV hood CP211):

Kaz

Need Girls

Diaclaimer

- Find measure for patient in packet

- Measure neck of patient

- Open Box take out bag

- show content of the package (remember to show the white connector within the CP924)

- the kit can be used in four application

  - NIV on the mechanical ventilator

  - CPAP on the flow driver

  - CPAP with included VentuPlus CP924 accessory

  - Say/show this is all listed into the leaflet within the package

- USE ON THE VENTILATOR (Maquet or the new Drager):

- Assembling the kit for use on the ventilator

  - Bits all on bed

  - Push Open Hood

  - Remove patient access port cap

  - Check valve

  - Connect rear straps

  - Connect filter to inspiratory side and then tubing.

  - Connect filter to expiratory side and then tubing.

  - Connect inspiratory and expiratory limbs (smoothbore) from the hood to the ventilator (no filters on hood in this example)

  - Show filter at machine end for protecting the ventilator and connect tubing

  - Turn on ventilator

  - Two nurses grip, show inside.

  - Lift over patient

  - Pull straps through and connect (guess)

  - Attach access port and pull valve

  - Adjust straps

- USE IN CPAP WITH THE FLOW DRIVER:

  - connect PEEP valve on the filter

  - connect PEEP + filter to the expiratory side of the hood

  - connect a smoothbore limb inspiratory side

  - connect the end of the tube to the InterFlow and open the flow

  - We can show a sequence adding a filter on the inspiratory side of the hood for reducing the noise and protecting the patient from environmental air

- Assembling the kit for use on the INTERFLOW

  - connect PEEP valve on the filter

  - connect PEEP + filter to the expiratory side of the hood

  - connect a smoothbore limb inspiratory side

  - connect the end of the tube to the InterFlow and open the flow

  - We can show a sequence adding a filter on the inspiratory side of the hood for reducing the noise and protecting the patient from environmental air

- USE IN CPAP WITH THE INCLUDED VENTUPLUS CP924:

- Assembling the kit for use on the INTERFLOW

  - connect PEEP valve on the filter

  - connect PEEP + filter to the expiratory side of the hood

  - connect a smoothbore limb inspiratory side

  - we can say here that it is possible to add a second filter in this position for reducing the noise and protecting the patient from environmental air

  - connect the VentuPlus CP924 to the end of the tube

  - connect the oxygen lines of the VentuPlus to the oxygen flow meter at the wall

  - show the table connected at the oxygen lines and open the oxygen taps

- NOW WE NEED TO SHOW HOW THE TABLES WORK – A BIT LIKE THE VENTUMAS VIDEO

- QUICK ROUND OF POSITIONING ON THE PATIENT