CaStar CPAP: Translation document

|  | English |  |
| --- | --- | --- |
| 01 | This video shows the process of setting up, fitting and removing a CaStar hood. Always refer to the Instructions for Use provided with each product. This video is not intended to replace the Instructions for Use. |  |
| 02 | CaStar CPAP Hood Fitting and removing. |  |
| 03 | A patient interface for CPAP that is comfortable, versatile and lightweight. |  |
| 04 | Available in six sizes to comprehensively cover the broadest range of patients from paediatrics to adults. |  |
| 05 | Size information is on the top and side of each box. |  |
| 06 | Some configurations include a tape measure on the outside so that the correct size can be selected without opening the box. |  |
| 07 | Inside the box you will find... |  |
| 08 | CaStar CPAP Hood |  |
| 09 | Under arm straps |  |
| 10 | Instructions for use |  |
| 11 | Ear plugs* (with some configurations) |  |
| 12 | Some configurations include ear plugs should you wish to use them. |  |
| 13 | Fitting the hood. |  |
| 14 | Open the bag using the perforated edge. |  |
| 15 | Reshape the helmet. |  |
| 16 | Remove the patient access port cap. |  |
| 17 | Check the smooth operation of the anti-asphyxiation valve and place to one side. |  |
| 18 | Check the smooth operation of the anti-asphyxiation valve and place to one side. |  |
| 19 | Attach the arm straps to the rear of the helmet to suit the patient. |  |
| 20 | Attach 22mm breathing system and PEEP valve. |  |
| 21 | Start the flow driver. |  |
| 22 | Two people firmly open the collar using their finger tips, and thumbs along the rigid ring so that the patient's head can pass through. |  |
| 24 | Raise the patients arms to ensure that the straps are tucked well under for strap sizing. |  |
| 25 | Fit the access port cap then pull the anti-asphyxiation button until the hood inflates. |  |
| 26 | Check the presence of pressure within the hood using the manometer. |  |
| 27 | If needed adjust the straps so that the rigid ring rests about 1cm away from the shoulders. |  |
| 28 | When positioned correctly you may fold and pin the front strap ends to enhance patient visibility. |  |
| 29 | The hood is now set up correctly. |  |
| 30 | You can use the access port for temporary patient access if needed. |  |
| 31 | Removing the hood |  |
| 32 | Remove the access port cap. |  |
| 33 | Undo the front straps. |  |
| 34 | Two people firmly open out the collar and remove the hood. |  |
| 35 | Stop the flow generator. |  |