|  | English V2 | English V4 |  |
| --- | --- | --- | --- |
| 01 | This video is a guide to fitting the VariFitTM NIV masks. In this example the VariFit NIV vented mask with anti-asphyxiation valve is shown together with the breathing tubing suitable for this mask. Always refer to the Instructions For Use provided with each mask before use. This video is not intended to replace the Intructions For Use. | Same |  |
| 02 | VariFit™ NIV mask | Same |  |
| 03 | Fitting guide | Same |  |
| 04 | Flexible Spacebar | Same |  |
| 05 | Active Headgear Connector | Same |  |
| 06 | Quick Release Hook | Same |  |
| 07 | Use the VariFit mask size guide to select the appropriate mask | Same |  |
| 08 | Detach one Velcro® strap by sliding the Velcro loop off the quick release hook, slip the headgear over the patient's head and centre the mask on the face. | Same |  |
| 09 | Velcro® is a registered trademark of Velcro Industries B.V | Same |  |
| 09@ | Practical tip: starting at the chin, roll the mask towards the forehead. | NEW |  |
| 10 | Take the detached Velcro strap and slide the Velcro loop back on to the quick release hook. | Same |  |
| 11 | Ensure the headgear straps are parallel and the headgear is fully extended and flat across the back of the head. | Same |  |
| 12 | Adjust the bottom headgear straps by using the Velcro tabs. After tightening, make sure that the Active Headgear Connector can move freely. | Same |  |
| 13 | Adjust the upper headgear straps by using the Velcro tabs. Do not over tighten the headgear, the Flexible Spacebar should not touch the forehead. | Same |  |
| 13 | Adjust the bottom headgear straps to make sure the bottom of the mask is sealed comfortably against the patient's face. | REMOVE |  |
| 14 | For optimal seal and comfort, the mask should be snug, but not tight. Adjust the tension by detaching the Velcro straps, tightening or loosening the straps, and reattaching the Velcro. | For optimal seal and comfort, the mask should be snug, but not tight. |  |
| 14 | Connect the mask to the positive airway pressure device tubing and turn the device on to the prescribed pressure. | Same |  |
| 15 | *breathing tube shown is only suitable for vented NIV masks. | Same |  |
| 16 | Quality innovation and choice. |  |  |
| 17 | facebook /intersurgical |  |  |
| 18 | www.intersurgical.com |  |  |
| 19 | @intersurgical |  |  |
|  | For youtube | Translations below are for web searches. |  |
|  | Fitting the Intersurgical VariFit NIV mask |  |  |
|  | Step by step guide showing how to fit the VariFit NIV mask to the patient. |  |  |
|  | NIV mask, Non-invasive ventilation |  |  |