i-flo fitting: Translation document

|  | English |  |
| --- | --- | --- |
| 01 | This video is a brief introduction to the Intersurgical i-flo™ High-Flow nasal cannula. The Intersurgical i-flo High-Flow nasal cannula must always be used in accordance with the Instructions for Use supplied with the device. This video is not a replacement for the Instructions for Use. |  |
| 02 | Intersurgical i-flo™ High-Flow nasal cannula |  |
| 03 | The i-flo™ nasal cannula is Intersurgical’s single use patient interface intended for the delivery of High Flow Oxygen Therapy (HFOT). |  |
| 04 | Fitting the i-flo |  |
| 05 | Sizing guides are supplied in the box. |  |
| 06 | Hold the guide up to the patient's nares to determine the correct size cannula. |  |
| 07 | The cannual prongs should not create a seal with the nares or press on the septum. |  |
| 08 | Select the correct size cannula. |  |
| 09 | Holding the strap away from the face, insert the nasal prongs into the patient's nares and lift the strap over their head. |  |
| 10 | Adjust the tension to ensure a comfortable fit. |  |
| 11 | The 180° rotating elbow can be moved left or right for convienient positioning. |  |
| 12 | Securing Option 1 Clip to patient clothing |  |
| 13 | The first time the clip is used, squeeze together until the two ends cross over and form the grip. |  |
| 14 | Squeeze and release to position appropriately on clothing. |  |
| 15 | Attach the breathing system to the i-flo swivel connector using a push and twist action. |  |
| 16 | Securing Option 2 Clip to lanyard |  |
| 17 | Open the lanyard, pass it behind the patient’s neck and re-clip. |  |
| 18 | Attach the lanyard to the fastening on the swivel connector. |  |
| 19 | i-flo’s adaptative design ensures the patient can receive treatment while resting comfortably. |  |