|  | nFlow 2021 update translation doc |  |
| --- | --- | --- |
|  | English |  |
| 01 | nFlow CosyFit |  |
| 02 | Infant nasal CPAP system |  |
| 03 | How to fit |  |
| 04 | Nasal mask sizing |  |
| 05 | The triangular outline should cover the patient nose contour but not exceeding. |  |
| 06 | Nasal prong sizing |  |
| 07 | Hold the interface size guide up to the patient´s nares to determine the best match nasal prong size. |  |
| 08 | Connect the selected patient interface to the nFlow CPAP device ensuring the correct orientation. |  |
| 09 | Select the appropriate bonnet size. |  |
| 10 | Fit the bonnet to the patient, ensuring the correct orientation of attachment pads. |  |
| 11 | Tie the top of the bonnet. |  |
| 12 | Reposition the tube support to achieve the correct distance from the patient’s nares to forehead. |  |
| 13 | Position the patient interface over / into the nares and ensure correct alignment. |  |
| 14 | Whilst holding the nFlow CPAP device in position bring the tube support into contact with bonnet attachment. |  |
| 15 | Gently attach the straps to the side attachment pads to ensure optimal patient comfort. |  |
| 16 | Caution: Overtightening the side straps may result in patient trauma. |  |
| 17 | Adjust the angle of the patient interface by compressing or extending the exhaust tube. |  |
| 18 | Intersurgical. Quality, innovation and choice. Interact with us. Intersurgical.com |  |