AccuPap™ Positive airway pressure therapy system

|  | English |  |
| --- | --- | --- |
| 1 | This video is a demonstration of the set-up for the AccuPAP™. It is not intended as a replacement for the instructions for use (IFU) provided with the products. Always follow the IFU before using the product. |  |
| 2 | AccuPAP™ Positive Airway Pressure Therapy System |  |
| 3 | The AccuPAP™ Provides positive airway pressure and is indicated for prevention or reversal of atelectasis in adult patient |  |
| 4 | Supply gas inlet connector Adjustable PEEP Disposable manometer port Gas tubing |  |
| 5 | Adjustable PEEP The PEEP can be adjusted by twisting the PEEP adjuster to suit the clinical requirements of the patient. The options are 5,10,15,20cmH20). |  |
| 6 | Disposable pressure manometer Can be used to monitor pressure within the AccuPAP system |  |
| 7 | Setting up the AccuPAP™ |  |
| 8 | AccuPAP™ In each pack you will find |  |
| 9 | AccuPAP positive airway pressure device PEEP Adjuster 5,10,15,20cmH20 Disposable pressure manometer* Gas supply tubing *Disposable pressure manometer is only supplied with product code 313-6001 |  |
| 10 | AccuPAP™ Set up |  |
| 11 | Connect one end of the gas tubing to a 0-15 LPM flow meter with a push and twist. Oxygen or Air can be used, whichever is clinically appropriate for the patient. |  |
| 12 | Connect the other end of the tubing to the AccuPAP™ supply gas inlet connection. |  |
| 13 | If pressure monitoring is not required, seal the port with the attached port cap. Then attach the mouthpiece with a push and twist. |  |
| 14 | Set initial flow rate to 8 LPM on the air or oxygen supply source flow meter. Adjust flow meter to desired level as needed. |  |
| 15 | Adjust the PEEP selector to the appropriate setting. The PEEP can be set at 5, 10,15,20cmH20 whichever is clinically appropriate |  |
| 16 | Instruct the patient to place the mouthpiece in their mouth making a seal. |  |
| 17 | The patient should inhale and exhale through the device. |  |
| 18 | Maintain breathing through the AccuPAP to accomplish clinical goals. |  |
| 19 | Several repetitions may be needed to ensure that individual patient needs are being met. |  |
| 20 | For complete product set-up details please refer to the Instructions For Use |  |