FiltaNeb™ script

|  | English |  |
| --- | --- | --- |
|  | This video is a brief guide introduction to the FiltaNeb™ ( Cirrus™2 nebuliser, Adult Intersurgical EcoLite™ non-vented mask kit with filter) The product must always be used in accordance with the Instructions for Use supplied with the device This video is not a replacement for the Instructions for Use. |  |
|  | FiltaNeb™ Cirrus™2 nebuliser, Adult Intersurgical EcoLite™ non-vented mask kit with filter |  |
|  | Fitting guide |  |
|  | FiltaNeb™ has been developed to help minimise potentially infected exhaled gases and emissions escaping during nebulisation therapy. |  |
|  | Inside the kit you will find . . . |  |
|  | Filter |  |
|  | Mask / t-piece /nebuliser |  |
|  | Second strap |  |
|  | Tube |  |
|  | How to fit |  |
|  | Attach the filter to the port on the T-piece using a push and twist action. |  |
|  | Unscrew the nebuliser cup from the nebuliser and add the medication to the nebuliser cup. |  |
|  | Note: maximum fill volume is 5ml |  |
|  | Screw the nebuliser cup to the nebuliser, a positive click will indicate the cup is closed. |  |
|  | Connect one end of the tube to the spigot connector at the bottom of the nebuliser, and connect the other end to the driving gas source. |  |
|  | Hold the mask to patient's face with one hand, and with the other hand take the elastic strap on the mask and secure below the ears. |  |
|  | Hold the mask in place and adjust the elastic strap by pulling on both sides to ensure a secure and comfortable fit. |  |
|  | Take the second strap and fit under the mask port, position the strap above the ears and tie at the back of the head to secure. |  |
|  | Turn on the gas flow, check to ensure there are no leaks around the mask, and adjust the straps accordingly. |  |
|  | Please refer to the Instructions for Use for further information. |  |
|  | Intersurgical |  |
|  | Quality, innovation and choice. |  |
|  | Interact with us |  |