|  | Text (English) | Translation |
| --- | --- | --- |
| 1 | This video is a demonstration of the set-up and configuration for the Intersurgical PEP Therapy product range. It is not intended to provide any clinical training or guidance, and it is not intended to be a replacement for the Instructions for Use (IFU) provided with the products. Always refer to the IFU before using the product. |  |
| 2 | Positive Expiratory Pressure (PEP) Therapy Device |  |
| 3 | The PEP therapy device consists of A PEP Resistor The clinician is able choose the size appropriate for the patient’s clinical needs A PEP T-piece with mouthpiece A PEP manometer connector A mask can be used in place of the mouthpiece if clinically appropriate |  |
| 4 | Nine colour coded resistor sizes available |  |
| 5 | Setting up |  |
| 6 | Using a push and twist action, attach the mouthpiece to the PEP T-piece |  |
| 7 | Using the same technique attach the PEP resistor to the PEP T-piece |  |
| 8 | The PEP T-piece has a one-way valve allowing air to flow through to the patient on inhalation, and when the patient breathes out their exhaled breath is directed down to the PEP resistor |  |
| 9 | One way valve Inhalation Exhalation |  |
| 10 | A face mask can be used |  |
| 11 | Using the PEP Therapy device with a pressure manometer |  |
| 12 | Attach the mouthpiece to the PEP T-piece using a push and twist action |  |
| 13 | Attach the PEP manometer connector to the PEP T-piece |  |
| 14 | Connect the PEP resistor to the PEP manometer connector |  |
| 15 | Then connect the pressure manometer |  |
| 16 | use a push and twist action to secure all connections |  |
| 17 | Note – pressure manometer is not included with this product |  |
| 18 | Interact with us |  |