|  | English | Your language |
| --- | --- | --- |
| 01 | This video is a brief introduction to the Sentri™ end tidal CO₂ monitoring oxygen masks and nasal cannula. The product must always be used in accordance with the Instructions for Use supplied with the device. This video is not a replacement for the Instructions for Use. |  |
| 02 | Sentri™ End tidal CO₂ monitoring How to use |  |
|  | The Sentri™ ranges are available with an adult mask and three sizes of nasal cannula. |  |
|  | Sentri™ enables the sampling of exhaled carbon dioxide in non-intubated patients during the administration of supplementary oxygen. There are also options available for connection to Microstream® capnography. |  |
|  | How to use |  |
|  | Connect the oxygen tube to the oxygen spigot on the oxygen mask with a push and twist. |  |
|  | Connect the other end of the oxygen tube to the flow meter with a push and twist and set the prescribed flow rate. |  |
|  | Position the mask on the patient’s face. |  |
|  | On large faces the mask fits under the chin. |  |
|  | Oxygen tube connection |  |
|  | ETCO₂ connection |  |
|  | On small faces a second inner seal fits under the chin. |  |
|  | Slide the strap over the back of the patient’s head, below the ears, and tighten as required. |  |
|  | The strap can also be positioned above the ears. |  |
|  | Attach the ETCO monitoring line to the ETCO₂ monitoring line port on the mask |  |
|  | Version with filter shown. |  |
|  | Attatch the ETCO2 monitoring line to the ETCO₂ monitoring line port on the mask. |  |
|  | The ETCO2 monitoring port can be rotated to suit the position of the capnography equipment. |  |
|  | O₂ connection |  |
|  | ETCO₂ connection |  |
|  | A Sentri™ nasal cannulae delivers oxygen through one prong and samples exhaled gas through the other. |  |
|  | With a Sentri™ nasal cannulae the ETCO₂ monitoring port is located at the end of the sampling line. |  |
|  | Intersurgical |  |
|  | Quality, Innovation and choice |  |
|  | Interact with us |  |
|  | www.intersurgical.com |  |